### Project Dashboard

CAM-M, Cambridge Bay Water Use Licence Renewal (148952)

Proposal Status: Conformity Determination Issued

##### Project Overview

Type of application: New

| Proponent name: | Jeremy Laflamme |
| --- | --- |
| Company: | Department of National Defence |

Schedule:

| Start Date: | 2019-03-31 |
| --- | --- |
| End Date: | 2029-03-31 |
| Operation Type: | Annual |

Project Description:

The North Warning System in Canada is a chain of unmanned radar sites that provides aerospace surveillance, established to detect and allow for an early response to potential threats entering North American air space. It is part of Canada’s North American Aerospace Defense Command (NORAD) agreement with the United States, and an essential capability in our efforts to maintain Canada's sovereignty. Raytheon Canada Limited has the contract with the Department of National Defence to operate and maintain the North Warning System radar site CAM-M, Cambridge Bay. CAM-M is situated in Nunavut on the south shore of Victoria Island in the Arctic Ocean. The site is about 35 m above sea level on a gently rising coastal plateau which has numerous lakes and ponds. The airport elevation is about 27 m above sea level. CAM-M has been manned since the 1950’s. It consists of a Long Range Radar (LRR) site and a Logistics Support Site (LSS). CAM-M’s facilities include site buildings with their integral mechanical and electrical systems, power generation system, fuel tanks, radar, antennas, satellite ground terminals, weather equipment, and roads. CAM-M, Cambridge Bay is staffed with an annual average site population of 18 to 22 people per day. Site numbers increase during the summer due to seasonal construction and occasional large groups of Third Party visitors. CAM-M is one of 11 LRRs of the North Warning System; the LRRs are located across the Yukon, Northwest Territories, Nunavut, and down the Labrador coast. The facilities are remotely monitored and controlled from North Bay on a 24/7 basis. The information they receive is automatically sent to the Canadian Air Defence Sector located at 22 Wing, CFB North Bay over a long-haul satellite communications network. This application is for the renewal of the CAM-M Nunavut Water Board water licence (3BC-CAM0919) with the following proposed changes: - Water use limit change from 10 m³/day to 3650 m³/year; - field testing of berm water (see Annex A4 for details); and - the potential to landfarm hydrocarbon impacted soils on-site (see annex A3 for details). The Nunavut Impact Review Board (NIRB) file number for the CAM-M water licence is 03DN119. The following documents are being submitted as the Water Licence Renewal Package for CAM-M: - Application for Water Licence Amendment 3BC-CAM0919 •Annex A1 - 090520 3BC-CAM0409 No Conformity Required-IMLE •Annex A2 - 090515 3BC-CAM0409 Screening Exempt-IMLE •Annex A3 - CAM-M Landfarm Plan •Annex A4 - Berm Water Sampling QAQC Plan •Annex A5 - 2017 KIT 001 BP 3BC-CAM0919 DND-Raytheon CAM-M •Annex A6 - NWB Licence Compliance Assessment - Remote Camp Supplementary Questionnaire 3BC-CAM0919 •Annex Q1 - 3BC-CAM0919 Executive Summary •Annex Q2 – Spill Contingency Plan •Annex Q3 – CAM-M Site Description •Annex Q4 – CAM-M Site Plan •Annex Q5 – Jet A1 SDS

Personnel:

| Persons: | 18 |
| --- | --- |
| Days: | 365 |

##### Project Map

List of all project geometries:

| ID | Geometry | Location Name |
| --- | --- | --- |
| 4648 | point | CAM-M, North Warning System site |

Planning Regions:

Qikiqtani

Affected Areas and Land Types

Municipal

Settlement Area

##### Project Land Use and Authorizations

Project Land Use

Other

Licensing Agencies

NIRB: Screening Decision Report

NWB: Type B Licence

NWB: Type B Licence

Other Licensing Requirements
No data found.

##### Material Use

Equipment

| Type | Quantity | Size | Use |
| --- | --- | --- | --- |
| Pickup Truck | 6 | 5.8x2x2.4 m | Transportation |
| Grader | 1 | 9.1x2.5x3 m | Road maintenance |
| Dozer | 1 | 5.8x3.4x3.6 m | earthworks, snow clearing |
| Water truck | 1 | 3x5x2.7 m | moving water |
| Loader | 1 | 7x3.8x2.3 m | Earthworks, snow clearing, moving materials. |
| Dump truck | 1 | 3x5x2.7 m | Earthworks, snow clearing |
| Snow plow | 2 | 3x5x2.7 m | Snowclearing |
| mini-excavator | 1 | 3x2.2x1 m | Earthworks |

Fuel Use

| Type | Container(s) | Capacity | UOM | Use |
| --- | --- | --- | --- | --- |
| Aviation fuel | 1 | 946300 | Liters | Power Generation |
| Aviation fuel | 1 | 75000 | Liters | Power generation |
| Aviation fuel | 1 | 4100 | Liters | Refuel Mobile Support Equipment |
| Aviation fuel | 2 | 69200 | Liters | Aviation |
| Aviation fuel | 2 | 946300 | Liters | Aviation/Power generation |

Hazardous Material and Chemical Use

| Type | Container(s) | Capacity | UOM | Use |
| --- | --- | --- | --- | --- |
| oil | 25 | 205 | Liters | maintenance |
| glycol | 2 | 205 | Liters | maintenance |
| paint | 1 | 205 | Liters | Site maintenance |
| Batteries | 1 | 205 | Liters | Power generation |

Water Consumption

| Daily Amount (m3) | Retrieval Method | Retrieval Location |
| --- | --- | --- |
| 10 | Pipeline / truck | Water supply lake |

##### Waste and Impacts

Environmental Impacts

Please see attached 'Annex Q2 - Spill Contingency Plan' and 'Annex Q4 - CAM-M Site Description'

Waste Management

| Waste Type | Quantity Generated | Treatement Method | Disposal Method |
| --- | --- | --- | --- |
| Combustible wastes | 20,000 kg | None | Municipality of Cambridge Bay Landfill |
| Hazardous waste | 50 Drums, 2 crates | None | Licensed Waste HAZMAT Disposal Facility (off-site) |
| Sewage (human waste) | 1,400,000 L | Prior to discharge, the waste is treated with a tertiary sewage treatment plant | Discharged to the environment |