Project Proposal Description: Drainage Improvements & Shore Protection, Pangnirtung, NU

The Department of Fisheries and Oceans, Small Craft Harbours Branch is responsible for keeping the Pangnirtung Harbour in safe working condition.  Small Craft Harbours is proposing the following work:

- Drainage Improvements:

  - To prevent harbour erosion in the spring melt, Small Craft Harbours is proposing to:

- Realign and deepen the existing drainage ditch

- Replace the existing culverts in the drainage ditch and resurface the road over new culverts with new gravel

- Realign and resurface the existing harbour access road and upper road with new gravel

- Remove the old culverts, resurface the parking area with new gravel, and install rock shore protection to prevent future erosion

- Install rock erosion control in new drainage ditch to ensure it functions properly in the spring

- Berm Extension:

  - To prevent the entrance channel of the harbour from infilling, Small Craft Harbours is proposing to:

- Construct a rock berm extension between harbour entrance channel and tidal drainage outlet

- Breakwater Upgrades:

  - To keep the existing harbour in good working condition, Small Craft Harbours is proposing to:

- Install armour stone on the existing breakwater in areas required

- Install additional gravel on fixed wharves where required

- Install rock for shore protection/erosion control on inner basin of the harbour