HEALY NWP TRANSIT – EQUIPMENT DETAILS:

| 4.3 Particulars of methods and scientific instruments: |  |  |
| --- | --- | --- |
| Types of samples and measurements | Methods to be used | Instruments to be used |
| Water vapour & gaseous C stable Isotopes- 18O, 2H, 13CO2 and 13CH4 . | Continuous Air and sea water sampling using pumps and analysers. | Picarro L2130i for water vapour isotopes and Picarro G2201i for CO2 and CH4. |
| Seawater Isotopes and Carbon: Isotopes- 14C, CH4, 18O, 2H Carbon: DOC, POC. | Continuous seawater sampling using flow through instruments and lab analysis. | Picarro L2130i for water isotopes; accelerator mass spectrometer for 14POC and 14DOC back in the UCIrvine lab. |
| Seawater properties- Temperature, Salinity, dissolved oxygen, fluorescence, PCO2, light transmission. | Continuous flow through seawater instrumentation- | Seabird Electronics: SBE 3 Temperature sensor SBE 43 dissolved oxygen sensor SBE 45 Micro TSG (thermosalinograph) Wetlabs: WetSTAR fluorometer (chlorophyll) C-Star Transmissometer |
| Seawater Particulate organic matter: POC and PN. | eawater filtration and laboratory analysis using high-temperature (~1000 C) combustion and gas chromatographic separation | Laboratory analysis using a NC2500 elemental analyser |
| Seawater masses of Oxygen and Argon used to estimate net biological oxygen production | Sample continuous flow seawater and collect discrete samples for Lab calibration. | Shipboard Mass spectrometer Pfeiffer Prisma Plus QMA 220. Optical Oxygen sensor Aanderaa 4330 . |
| Expendable probe CTD data | Expendable probes while underway | Lockheed Martin XCTD-1 |
| Earthâ€™s Gravity field | Shipboard underway Gravimeters | Bell BGM3 and Dynamic Gravity Systems AT1M underway marine gravimeter |
| Sea floor mapping | Hull mounted shipboard Multibeam echo sounder | Kongsberg EM122 Multibeam |
| Ocean Current velocity and direction | Hull mounted echo sounders. Acoustic Doppler current profilers. ADCPs | Teledyne Marine Ocean Surveyor 150 (OS150) and Ocean Surveyor 75 (OS75) |
| 2D image of sediment stratigraphy. | Hull mounted echosounder. | Knudsen CHIRP 3260 with 12kHz and 3.5 kHz frequency transducers |
| Sky radiance and ocean surface reflectance for remote sensing application. | Ship mounted continuous sampling instrument. | hyperspectral radiometer (HyperSAS). |
| eDNA analysis of Seawater focusing on zooplankton | Filtering seawater from underway seawater system and preserving filters for post cruise laboratory analysis. | Laboratory analysis of filters using Pelagic Ecosystems Labâ€™s quantitative Polymerase Chain Reaction (qPCR) instrument |
| Meteorological measurements | Shipboard sensors for continuous measurements | wind speed and direction: Metek usonic2 air temperature: RM Young 41342VC air temp/relative humidity: Vaisala HMP110 barometer: RM young 61302V surface PAR: Biospherical Instruments QSR 2200 radiometer pyranometer: Eppley PSP radiometer pyrgeometer: Eppley PIR |
| seawater pCO2 system | Sampling of Continuous flow Seawater | Seawater/air equilibrator, LI-COR infrared gas analyzer, gas dryer, pressure sensor |
| Atmospheric CO2 | Air intake mounted on the Bow of Healy for continuous sampling | Picarro G2301 or similar |