Spills Contingency Planning and Reporting

Initial Action in the Event of a Fuel Spill

In the event that a fuel spill has occurred, below is a list of procedures and suggested course of action of the first person on the scene who has detected a problem.

- Be alert and consider your safety first. If possible, identify the product spilled;

- Assess the hazards to persons in the vicinity of the spill and alert or take appropriate evacuation measures if needed (e.g., eliminate sources of ignition);

- If possible, control danger to human life;

- Assess whether the spill can be readily stopped or brought under control;

- If safe to do so, and if possible, try to stop the flow of material (e.g., stop fuelling, shut off valve (if present), manoeuvring a leaking drum);

- Gather information on the status of the situation;

- Report the spill without delay to the spill response person/team affiliated with the project and ensure that where applicable the government is notified at the same time according to via the 24 Hour Spill Report Line for Nunavut

Spill Procedure

In the case of any spill or other environmental emergency, it is necessary to react in the most immediate safe and environmentally responsible manner.  No spill or incident is so minor that it can be ignored, and every spill must be reported.

Chain of command

- Immediately notify and report the 24-hour Spill Report Line at (867) 920-8130, the CIRNAC Water Resource Officer at (867) 975-4550, and Kivalliq Inuit Association at (867) 645-5731.

- A Spill Report Form (Appendix 1) is filled out as completely as possible before or after contacting the 24-hour Spill Report Line.  A copy of the guidelines for completing the Spill Report form is found in Appendix II.

- Notify Martin McCurdy, GEM-GeoNorth Science Project Officer at (613) 859-5835.

Contacts for spill response/assistance and further reporting

Nunavut & NWT 24-hour Spill Report Line

(867) 920-8130

Water Resources Inspector for Crown-Indigenous Relations & Northern Affairs Canada (formerly INAC)

Iqaluit, NU

(867) 975-4550

24-hour pager (867) 766-3737

GN-Dept of Environment (DOE)

(867) 975-7700

Manager of Pollution Control and Air Quality

(867) 975-7748

Kivalliq Inuit Association, Lands Department

32-4 Sivulliq Avenue

340 Rankin Inlet, Nunavut XOC-0G0

Ph.: (867) 645-5731

Toll free 1-800-220-6581

FAX: (867) 645-2348

Nunavut Water Board, Head Office Gjoa Haven

(867) 360-6338

Fisheries and Oceans Canada, Habitat Impact Biologist

(867) 979-8007

Government of Nunavut Emergency Health information

(867) 975-5910

GN-Dept of Environment (DOE)

(867) 975-7700

Manager of Pollution Control and Air Quality

(867) 975-7748

Kivalliq Inuit Association, Lands Department

32-4 Sivulliq Avenue

340 Rankin Inlet, Nunavut XOC-0G0

Ph.: (867) 645-5731

Toll free 1-800-220-6581

FAX: (867) 645-2348

Program Manager

Michel Plouffe

GEM-GeoNorth Program Manager

Geological Survey of Canada/Commission géologique du Canada

Land & Minerals Sector/Secteur des terres et minéraux

Natural Resources Canada

239-601 Booth Street, Ottawa, Ontario K1A 0E8

Telephone: (613) 218-4728

E-mail: michel.plouffe@NRCan-RNCan.gc.ca

Project Manager

Martin W. McCurdy

GEM-GeoNorth Project Officer

Geological Survey of Canada/Commission géologique du Canada

Land & Minerals Sector/Secteur des terres et minéraux

Natural Resources Canada

279-601 Booth Street, Ottawa, Ontario K1A 0E8

Telephone: (613) 859-5835

E-mail: martin.mccurdy@NRCan-RNCan.gc.ca

Project Officer

Martin W. McCurdy

GEM-GeoNorth Project Officer

Geological Survey of Canada/Commission géologique du Canada

Land & Minerals Sector/Secteur des terres et minéraux

Natural Resources Canada

279-601 Booth Street, Ottawa, Ontario K1A 0E8

Telephone: (613) 859-5835

E-mail: martin.mccurdy@NRCan-RNCan.gc.ca

### Spill Equipment

Spill kits will be will be carried in the helicopter at all times. Spill kits consist of:

- Heavy PVC tarp, impermeable to Jet A aviation and gasoline spills, sized in accordance with fuel containers (12’x14’ for drums of Jet A, 4’x4’ for plastic jerry cans, if used)

- Aluminum stakes to secure impermeable tarp to ground

- Particulate absorbent

- Petroleum sorben pads

- 2 pair PVC gloves

- 2 pair safety goggles

- Disposable bags

- 1 shovel

- Fire extinguisher

Spill Response Actions for Aviation Fuel

Take action only if safety permits.  Stop the source flow if safe to do so and eliminate all ignition sources. Never smoke when dealing with these types of spills.

On Land

- Build a containment berm using soil material or snow and place a plastic tarp at the foot of the berm for easy capture of the spill after all vapours have dissipated.

- Remove the spill by using absorbent pads or excavating the soil, gravel or snow.

- Remove spill splashed on vegetation using particulate absorbent material.

- Contact regulatory agencies for approval before commencing with the removal of any soil, gravel or vegetation.

On Muskeg

- Do not deploy personnel and equipment on marsh and vegetation.

- Remove pooled gasoline or Jet A with sorbent pads and/or skimmer.

- Flush with low pressure water to push toward collection point.

- On advice from regulatory agencies, burn only in localized areas, e.g. trenches, piles or windrows.

- Do not burn if root systems can be damaged (low water table).

- Minimize damage caused by equipment and excavation.

On Water

- Contain spill as close to release point as possible.

- Use containment boom to capture spill for recovery after vapours have dissipated.

- Use absorbent pads to capture smaller spills.

- Use skimmer for larger spills.

On Snow and Ice

- Build a containment berm around spill using snow.

- Remove the spill using absorbent pads or particulate sorbent material/

- The contaminated ice and snow must be scraped and shovelled into plastic buckets with lids, 205 litre drums, or polypropylene bags.

Storage and Transfer

All contaminated water, ice, snow, soil and clean-up supplies will be stored in closed, labelled containers.  All containers will be stored in a well-ventilated area away from incompatible materials.

Disposal

Any contaminated material will be shipped to an appropriate and approved disposal facility. The DOE monitors the movement of hazardous wastes from generators, carriers to receivers, through a tracking document (Waste Manifest).  A waste manifest will accompany all movements.