KIVALLIQ INUIT ASSOCIATION

LAND COMPLIANCE SUBMISSION

Esker GOLD PROJECT

2026 DIAMOND DRILLING PROGRAM

Proposed Land Use Activities and Disturbance Summary

Proponent:
Guardian Exploration Inc.

Project Location:
South Henik Lake Area

Kivalliq Region, Nunavut
Inuit Owned Lands

Submission Type:
Land Compliance Update
2026 Seasonal Diamond Drilling Program

Date of Submission:
01 March 2026

Submitted by:

Graydon Kowal
Guardian Exploration Inc.
gkowal@guardianex.com

DOCUMENT CONTENTS

This submission includes:

• Purpose and Scope of 2026 Activities
• Detailed Land Disturbance and Infrastructure Summary
• Drill Pad Disturbance Appendix
• Sump Volumes and Active Management Summary
• Temporary Infrastructure Footprint Summary
• Fuel Storage and Containment Description
• Environmental Protection and Reclamation Commitments
• Wildlife Mitigation Commitments
• Archaeological Chance Find Procedure
• Community Engagement Documentation
• Supporting Maps and Spatial Data

1.0 Scope of 2026 Activities

The proposed 2026 program includes:

- Up to 8 drill pads (approximately 15 m × 15 m each);

- Multiple drill holes from individual pad areas (commonly two and up to four per pad);

- Helicopter-supported access (no road construction);

- Temporary seasonal core logging facility and camp;

- Secondary-contained fuel storage (maximum 4,000 litres on site at any given time);

- Closed-loop water recirculation with conservative withdrawal limits;

- Wildlife mitigation measures including 500 metre caribou setback;

- Sump excavation and backfilling;

- Progressive reclamation of drill pads.

Certain exploration activities approved for the 2025 field season that were not completed may be undertaken during the 2026 season. These activities remain within the originally described scope and do not expand the project footprint.

2.0 DETAILED LAND DISTURBANCE AND INFRASTRUCTURE

2.1 Drill Pads

Maximum number of drill pads: 8
Approximate dimensions per pad: 15 m × 15 m (225 m²)
Maximum cumulative pad disturbance: 1,800 m²

Multiple drill holes (commonly two and up to four) may be completed from a single pad area in order to reduce overall surface disturbance and limit the need for additional pad construction.

Minor leveling may be required to ensure safe drill operation and will be achieved using timber cribbing, locally available rock, and limited redistribution of existing soil materials.

No road construction, corridor clearing, or blasting is proposed.

| Drillhole Name | Easting | Northing | Elevation | Azimuth | Dip | Target Depth |
| --- | --- | --- | --- | --- | --- | --- |
| GEE-26-01 | 592343 | 6829538 | 190 | 336 | 60 | 175 |
| GEE-26-02 | 592343 | 6829538 | 190 | 336 | 75 | 250 |
| GEE-26-03 | 592436 | 6829548 | 190 | 275 | 45 | 200 |
| GEE-26-04 | 592436 | 6829548 | 190 | 275 | 60 | 200 |
| GEE-26-05 | 592400 | 6829650 | 190 | 240 | 45 | 200 |
| GEE-26-06 | 592400 | 6829650 | 190 | 240 | 60 | 200 |
| GEE-26-07 | 592325 | 6829700 | 190 | 205 | 45 | 200 |
| GEE-26-08 | 592325 | 6829700 | 190 | 205 | 60 | 200 |
| GER-26-09 | 619919 | 6837286 | 172 | 0 | 50 | 200 |
| GER-26-10 | 619919 | 6837286 | 172 | 180 | 50 | 200 |
| GER-26-11 | 620030 | 6837253 | 172 | 0 | 50 | 200 |
| GER-26-12 | 620030 | 6837253 | 172 | 180 | 50 | 200 |
| GER-26-13 | 620199 | 6837411 | 179 | 0 | 50 | 200 |
| GER-26-14 | 620199 | 6837411 | 179 | 180 | 50 | 200 |
| GER-26-15 | 620189 | 6837940 | 184 | 0 | 50 | 200 |
| GER-26-16 | 620189 | 6837940 | 184 | 180 | 50 | 200 |

2.2 Sumps

Up to three (3) drill sumps may be active at any given time.

Approximate volume per sump: 13.5 m³.

Sumps will be excavated using a small mini track hoe confined to drill pad areas. Sumps will:

- Contain drill cuttings;

- Facilitate settlement of solids;

- Support closed-loop recirculation of drill water.

Following completion of drilling at each location, sumps will be backfilled and stabilized to restore surface contours as closely as practicable.

2.3 Seasonal Infrastructure

Three potential camp locations have been identified in case one or two are unfeasible.

Temporary infrastructure associated with the 2026 drilling program includes:

- Core logging facility tent frame (approximately 14 ft × 16 ft or 16 ft × 20 ft);

- Kitchen tent (approximately 16 ft × 20 ft)

- Drillers dry (approximately 16 ft × 20 ft)

- First aid tent (approximately 16 ft × 20 ft)

- Sleeping tents (4X approximately 16 ft × 20 ft)

- Office tent (approximately 16 ft × 20 ft);

- Storage/maintenance tent (approximately 16 ft × 20 ft)

- Outhouse structure;

- Secondary-contained fuel storage area (approximately 5 m × 5 m footprint).

These installations are seasonal, non-permanent structures without fixed foundations.

It is anticipated that the camp tent frames may remain in place at the conclusion of the 2026 field season for reuse during subsequent exploration programs. These structures will be secured and maintained in a stable and environmentally responsible condition.

2.4 Fuel Storage and Handling

Maximum diesel stored on site at any given time: 2,500–4,000 litres.
Estimated total seasonal diesel usage: approximately 40,000 litres.

Fuel will be stored within secondary containment and managed in accordance with the project Spill Contingency Plan.

2.5 Access and Equipment

Access to drill sites will be primarily by helicopter. Helicopters will place drill rigs directly onto prepared pads.

Limited ATV use may occur along existing natural pathways for movement of supplies. No new access corridors or roads will be constructed.

A mini track hoe previously utilized during the 2025 field season will be used only as required for minor pad leveling and sump excavation.

All disturbance associated with the 2026 program remains confined within the Esker mineral claims.

3.0 ENVIRONMENTAL PROTECTION AND RECLAMATION COMMITMENTS

3.1 Wildlife Protection

Guardian Exploration Inc. will implement the following wildlife protection measures during the 2026 drilling program:

- A minimum 500 metre setback from caribou;

- Immediate pause of drilling activities if caribou herds enter the immediate operational area;

- Engagement of a local Inuit wildlife monitor during active drilling;

- Minimization of helicopter overflights in areas where wildlife are observed;

- Secure storage of food, fuel, and waste to prevent wildlife attraction;

- Prohibition of hunting by project personnel.

These measures are intended to minimize disturbance and maintain compatibility with traditional land use activities.

3.2 Water and Aquatic Protection

Drilling operations will utilize a closed-loop recirculating water system.

Freshwater withdrawal will not exceed 50 m³/day (conservative planning maximum) and 2,500 m³ total seasonal withdrawal.

No discharge of drill water to natural water bodies is proposed.

Intake hoses will be screened to prevent fish entrainment and positioned to avoid sediment disturbance.

3.3 Waste Management

Waste generated during the drilling program will be managed in accordance with best industry practices and the Spill Contingency Plan.

Measures include:

- Containment of drill cuttings within excavated sumps;

- Backfilling and stabilization of sumps following drilling;

- Removal or appropriate disposal of solid waste;

- Secure storage of fuel within secondary containment;

- Proper handling of used oil and hydrocarbon-contaminated materials.

3.4 Reclamation and Site Stabilization

Upon completion of drilling activities at each pad:

- Drill equipment will be removed by helicopter;

- Surface materials will be redistributed as required to restore natural contours;

- Sumps will be backfilled and stabilized;

- Areas will be inspected for erosion, contamination, or safety hazards.

Temporary structures remaining for reuse will be secured and maintained in stable condition.

4.0 MONITORING

4.1 Ongoing Monitoring and Compliance

Guardian Exploration Inc. will:

- Inspect disturbed areas during demobilization;

- Address any observed erosion or instability;

- Provide updates to KIA as required under reporting obligations.

The 2026 program is designed to ensure that land disturbance remains temporary, controlled, and consistent with responsible mineral exploration on Inuit Owned Lands.

4.2 Financial and Liability Commitments

Guardian Exploration Inc. confirms that it maintains appropriate corporate capacity to:

- Implement reclamation measures described herein;

- Address spill response and environmental remediation if required;

Proof of insurance and related documentation will be provided to KIA upon request.

4.3 Commitment to Ongoing Collaboration

Guardian Exploration Inc. values its working relationship with the Kivalliq Inuit Association and remains committed to:

- Transparent communication;

- Respect for Inuit Owned Lands and traditional land use;

- Early notification of material operational changes;

- Responsible exploration practices consistent with community expectations.

5.0 APPENDICES

The following appendices are provided:

APPENDIX A – PROJECT LOCATION AND DISTURBANCE MAPS

Tenure Distribution Map
Showings Location Map
Proposed Camp Locations

Drillhole Location Map – Esker

Drillhole Location Map - River  
Proposed Camp Layout
KML Spatial Files

All map coordinates are in UTM NAD83 Zone 14N.

APPENDIX B – 2026 DISTURBANCE SUMMARY

Drill Pad Disturbance Table
Sump Quantities and Volumes
Temporary Infrastructure Footprint Summary
Fuel Storage and Containment Description

APPENDIX C – ENVIRONMENTAL MANAGEMENT DOCUMENTS

Spill Contingency Plan
Wildlife Mitigation Commitments Summary
Archaeological Chance Find Procedure
Reclamation and Stabilization Summary

APPENDIX D

2025 KVL125B04_250808_Approval letter

KIA_KVL125B04_250808_License